### Technical Lab Blueprint: Horizon Custom Fabrication (April 21 Session)

This blueprint summarizes our successful deployment from the April 21 session. I’ve synthesized the messy whiteboard notes and CLI logs into a structured guide. I finally had an "aha!" moment regarding how L3 routing actually interlocks with L2 tagging on our Extreme 5320 switches, and I’ve documented those lessons in the "Vault" at the end. This is the "why" behind the CLI "what."

#### 1. Architectural Overview: Primary/Secondary (L3/L2) Design

The Horizon Custom Fabrication network uses a hierarchical design to centralize intelligence while keeping the access layer simple. We utilize two Extreme 5320 switches:

- SW1 (Primary/L3 Core):  This is the "brain." It owns all Layer 3 Gateways (SVIs), manages the internal DHCP server, and handles inter-VLAN routing logic.

- SW2 (Secondary/L2 Access):  This switch acts as a transparent extension. It provides extra ports for wired clients and APs but makes no routing decisions. It simply passes tagged frames up to SW1.

##### Core VLAN Configuration

VLAN ID,Name,IP Subnet,Gateway,Purpose

10,Corporate Wired,10.10.0.0/24,10.10.0.1,Hardwired Desktop/Server Access

20,Corporate Wireless,10.20.0.0/24,10.20.0.1,Internal Employee WiFi

30,Guest Wireless,10.30.0.0/24,10.30.0.1,Public/Guest Internet Access

#### 2. Physical Port Mapping & Infrastructure Logic

I realized that standardizing port layouts saved us an hour of troubleshooting when we were swapping APs. We use the same physical layout on both switches for consistency.

- Port 1 (Uplink):  Connects SW1 to the Quantum Fiber modem. It resides on the Default VLAN (VLAN 1) and is untagged.

- Port 3 (AP Trunk):  Connects to the AP3000 Access Points. This requires  VLAN 1 untagged  (The "AP Management Lifeline") so the AP can grab an initial 192.168.0.x IP from the modem to talk to the cloud.  VLANs 20 and 30 are tagged  to carry the actual SSID traffic.

- Port 5 (Wired Access):  Standard client access port. Configured as  Untagged on VLAN 10 .

- Port 10 (Inter-Switch Trunk):  The backbone between SW1 and SW2. All data VLANs (10, 20, 30) are  Tagged  here.  Note:  VLAN 1 must be removed from this port to prevent loops.

#### 3. SW1 (Primary/L3) Verified CLI Configuration Script

This script transforms SW1 into our central router and DHCP authority.

# === VLAN Creation and Physical Port Assignment ===

create vlan VLAN_10_Corporate_Wired tag 10

configure vlan VLAN_10_Corporate_Wired add ports 5 untagged

configure vlan VLAN_10_Corporate_Wired add ports 10 tagged

create vlan VLAN_20_Corporate_Wireless tag 20

configure vlan VLAN_20_Corporate_Wireless add ports 3 tagged

configure vlan VLAN_20_Corporate_Wireless add ports 10 tagged

create vlan VLAN_30_Guest_Wireless tag 30

configure vlan VLAN_30_Guest_Wireless add ports 3 tagged

configure vlan VLAN_30_Guest_Wireless add ports 10 tagged

# === AP Management Lifeline ===

# Necessary for APs to reach the modem for onboarding (See Section 5)

configure vlan Default add ports 3 untagged

# === L3 SVI and Routing Configuration ===

# CRITICAL: See "The Karl Rule" in Section 8 for ipforwarding

configure vlan VLAN_10_Corporate_Wired ipaddress 10.10.0.1/24

enable ipforwarding vlan VLAN_10_Corporate_Wired

configure vlan VLAN_20_Corporate_Wireless ipaddress 10.20.0.1/24

enable ipforwarding vlan VLAN_20_Corporate_Wireless

configure vlan VLAN_30_Guest_Wireless ipaddress 10.30.0.1/24

enable ipforwarding vlan VLAN_30_Guest_Wireless

# === Internal DHCP Server Configuration ===

# Port 10 is included for "Reach-Back" logic to serve SW2 clients

configure vlan VLAN_10_Corporate_Wired dhcp-address-range 10.10.0.100 - 10.10.0.200

configure vlan VLAN_10_Corporate_Wired dhcp-options default-gateway 10.10.0.1 dns-server primary 8.8.8.8

enable dhcp ports 5,10 vlan VLAN_10_Corporate_Wired

configure vlan VLAN_20_Corporate_Wireless dhcp-address-range 10.20.0.100 - 10.20.0.200

configure vlan VLAN_20_Corporate_Wireless dhcp-options default-gateway 10.20.0.1 dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_20_Corporate_Wireless

configure vlan VLAN_30_Guest_Wireless dhcp-address-range 10.30.0.100 - 10.30.0.200

configure vlan VLAN_30_Guest_Wireless dhcp-options default-gateway 10.30.0.1 dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_30_Guest_Wireless

#### 4. SW2 (Secondary/L2) Verified CLI Configuration Script

SW2 is much simpler. It just needs to know the VLAN IDs and which ports to tag.  Do not add IP addresses or enable ipforwarding here.

# === VLAN Creation (Mirror IDs from SW1) ===

create vlan VLAN_10_Corporate_Wired tag 10

configure vlan VLAN_10_Corporate_Wired add ports 5 untagged

configure vlan VLAN_10_Corporate_Wired add ports 10 tagged

create vlan VLAN_20_Corporate_Wireless tag 20

configure vlan VLAN_20_Corporate_Wireless add ports 3 tagged

configure vlan VLAN_20_Corporate_Wireless add ports 10 tagged

create vlan VLAN_30_Guest_Wireless tag 30

configure vlan VLAN_30_Guest_Wireless add ports 3 tagged

configure vlan VLAN_30_Guest_Wireless add ports 10 tagged

# === AP Management Lifeline ===

configure vlan Default add ports 3 untagged

#### 5. ExtremeCloud IQ (XIQ) Wireless Policy Configuration

The AP3000s are managed via XIQ. For the SSIDs to work, the Cloud Policy must map the Wireless traffic to our switch VLAN tags.| SSID | Security | VLAN | Client Isolation || ------ | ------ | ------ | ------ || Horizon-Corp | PSK | 20 | Off || Horizon-Guest | OWE (Enhanced Open) | 30 | On |

The AP Management Lifeline:  I discovered that APs boot as "blank slates." They send untagged DHCP requests. By keeping the Default VLAN (VLAN 1) untagged on Port 3, the APs reach the modem, get a 192.168.0.x IP, and can call home to XIQ. If you tag VLAN 1, the APs will never onboard.

#### 6. Security & Traffic Strategy (ACLs & Guest Isolation)

We use a two-pronged approach to ensure Guest users can’t touch Corporate data.

##### 6.1 Layer 3 Isolation (Inter-VLAN)

I configured an ACL on SW1 to block Guest traffic from jumping subnets. In EXOS, we define this policy and apply it to the ingress of the Guest VLAN.Policy Logic (Guest_Isolation.pol):

- Deny  10.30.0.0/24 to 10.10.0.0/24

- Deny  10.30.0.0/24 to 10.20.0.0/24

- Permit All Others  (Allows Internet access)CLI Implementation:

# Create the ACL policy file logic

edit policy Guest_Isolation.pol

# (Inside editor)

entry block_wired { if { destination-address 10.10.0.0/24; } then { deny; } }

entry block_wireless { if { destination-address 10.20.0.0/24; } then { deny; } }

entry permit_internet { if { } then { permit; } }

# Apply the policy to the Guest VLAN

configure vlan VLAN_30_Guest_Wireless access-profile Guest_Isolation

##### 6.2 Layer 2 Isolation (Intra-VLAN)

L3 ACLs can't stop guests on the same AP from talking to each other. For that, we enable  Client Isolation  in XIQ on the Guest SSID. This prevents peer-to-peer communication within VLAN 30.

#### 7. QoS (Quality of Service) Prioritization

On the trunk links (Ports 3 and 10), bandwidth contention is real. We use EXOS QoS profiles to ensure Corporate traffic is always serviced first.

- Corporate Wireless (VLAN 20):  Mapped to qp6 (High Priority).

- Guest Wireless (VLAN 30):  Mapped to qp1 (Best Effort).Implementation Note:  In EXOS, these markings are applied directly to the VLANs, ensuring that as traffic from VLAN 20 enters the trunk, it is placed in a higher-priority egress queue than Guest traffic. configure qosprofile qp6 vlan VLAN_20_Corporate_Wireless configure qosprofile qp1 vlan VLAN_30_Guest_Wireless

#### 8. The "Alex" Vault: Lessons Learned & Critical Rules

These were my biggest "aha!" moments from the April 21 session.

- The Karl Rule:  I learned the hard way that ipforwarding MUST be enabled on every routed VLAN on the Primary switch. Without it, the switch has an IP on the VLAN, but it acts like a wall—it will not move packets between subnets, and the gateway remains "silent" to other VLANs.

- The DHCP Reach-Back Logic:  On SW1, we must enable DHCP on  Port 10 . This was a breakthrough: it tells the Primary switch to "listen" for DHCP discovery packets coming across the trunk from clients physically connected to SW2.

- The Loop Triangle Risk:  We found that leaving the Default VLAN (VLAN 1) on the trunk (Port 10) while it's also connected to the modem creates a broadcast storm risk. We manually fixed this with: configure vlan Default delete ports 10.

#### 9. External Integration: Modem Static Routing

The Quantum Fiber modem is "VLAN-unaware." It only knows about its own 192.168.0.x subnet. To get internet traffic back to our clients, we must tell the modem that SW1 is the gateway for our internal networks.

- Next Hop IP (SW1 Management):  192.168.0.28

- Route 1 (Corporate Wired):  10.10.0.0/24 via 192.168.0.28

- Route 2 (Corporate Wireless):  10.20.0.0/24 via 192.168.0.28

- Route 3 (Guest Wireless):  10.30.0.0/24 via 192.168.0.28