I am going to redo this lab aligned with the document attached. Starting from scratch.

1:05 PM

## Claude responded: Got it — clean slate, aligned to the Horizon Custom Fabrication requirements.

Extract network requirements document

Extract network requirements document

Got it — clean slate, aligned to the Horizon Custom Fabrication requirements. Let me summarize what the document defines so we're on the same page:

What's given:

- 2 x Extreme EXOS switches (SW1 = primary/L3, SW2 = secondary/L2)

- 2 x APs (PlatformOne managed)

- Three VLANs:

  - VLAN 10 — Corporate Wired

  - VLAN 20 — Corporate Wireless

  - VLAN 30 — Guest Wireless

- Corporate SSID: PSK authentication

- Guest SSID: OWE (open/enhanced open)

- SW1 owns all SVIs (gateway IPs)

- SW2 is pure L2 access switch

Before we start building configs, I want to go Socratic on the discovery questions first — that's actually listed as a deliverable in the document.

Question 1: Looking at the three VLANs — Corporate Wired (10), Corporate Wireless (20), and Guest (30) — what IP subnets would you assign to each one? Pick your addressing scheme and tell me your reasoning.

## You said: 10.

10.10.0.1, 10.20.0.1, 10.30.0.1 - simple and straightforward with the gateways.

1:06 PM

## Claude responded: Perfect — clean and logical.

Perfect — clean and logical. The VLAN number maps directly to the third octet, making it easy to read and troubleshoot. So:

- VLAN 10 — Corporate Wired: 10.10.0.0/24, gateway 10.10.0.1 (on SW1)

- VLAN 20 — Corporate Wireless: 10.20.0.0/24, gateway 10.20.0.1 (on SW1)

- VLAN 30 — Guest Wireless: 10.30.0.0/24, gateway 10.30.0.1 (on SW1)

Question 2: Looking at the physical topology — you have SW1 (primary), SW2 (secondary), and two APs.

How would you distribute the APs between the two switches? And which VLANs would each AP carry? Remember each AP needs to broadcast both the Corporate SSID (VLAN 20) and the Guest SSID (VLAN 30) — so both APs carry both wireless VLANs.

What does that mean for how you configure the AP ports on each switch — access or trunk? And why?

## You said: AP1 - SW1, carries both VLAN20 and VLAN30.

AP1 - SW1, carries both VLAN_20 and VLAN_30. AP2-SW2, carries both VLAN_20 and VLAN_30. I will put the APs in port-3, the wired in port-5, trunk in port-10, uplink-port1

1:08 PM

## Claude responded: Exactly right.

Exactly right. Since each AP carries two SSIDs (two VLANs), the AP ports must be trunk ports — not access. An access port can only carry one VLAN, and you need both VLAN 20 and VLAN 30 tagged to each AP.

So your port layout is clean and consistent across both switches:

| Port | Role | Type | VLANs |
| --- | --- | --- | --- |
| Port 1 | Uplink to ISP/modem | — | TBD |
| Port 3 | AP1 (SW1) / AP2 (SW2) | Trunk | 20, 30 |
| Port 5 | Wired corporate device | Access | 10 |
| Port 10 | SW1 ↔ SW2 trunk | Trunk | 10, 20, 30 |

Question 3: Port 1 — the uplink. On SW1, that's your connection to the ISP/modem.

What VLAN does the uplink port belong to? And does it need to be tagged or untagged? Think about what the modem expects to see coming in from SW1.

## You said: Uplink on VLAN_1, Default.

Uplink on VLAN_1, Default. Untagged.

1:09 PM

## Claude responded: Correct.

Correct. The modem expects plain untagged Ethernet — it has no concept of VLANs. So port 1 on SW1 sits on the default VLAN 1, untagged. Clean.

Question 4: Now let's talk about the SW1 ↔ SW2 trunk on port 10.

You said it carries VLANs 10, 20, and 30. But here's a detail worth thinking about — what about the native/untagged VLAN on that trunk?

On an 802.1Q trunk, there's typically one VLAN that travels untagged. On your trunk between SW1 and SW2, should any VLAN be untagged on port 10? Or should all three VLANs (10, 20, 30) be tagged?

Think about what devices are on the other end of that trunk — it's switch-to-switch, not switch-to-end-device. What does that tell you about whether you need an untagged VLAN on that port?

## You said: ALL there VLANs on Port-10 must be tagged.

ALL there VLANs on Port-10 must be tagged.

1:09 PM

## Claude responded: Correct.

Correct. Switch-to-switch trunk — everything tagged. No untagged VLAN needed on port 10 because there's no end device expecting untagged frames on either side. Both switches understand 802.1Q tags.

Question 5: Now let's talk about DHCP. The document says no strict requirements — configure however is optimal.

You have three VLANs. Where should DHCP live? You have two options:

- Option A: Run DHCP directly on SW1 (the 5320 has a built-in DHCP server) — one per VLAN

- Option B: Use the upstream modem/ISP router as the DHCP server for all VLANs

Which would you pick for this lab, and why? Think about what you already know about how the modem handles non-native VLANs — we ran into this problem before in your production environment.

## You said: I have been using option-B to date.

I have been using option-B to date. With it, I have had to configure static-routes from my modem to the SW1 and SW2 directly to get DHCP, configuring the gateways for the two VLANs and then 8.8.8.8 for DNS. In this case, we can use optionA but I am looking to figure out how? Previously, configure vlan VLAN_70 dhcp-options default-gateway 10.70.0.1 and similar with dns server primary to 8.8.8.8 where I set the static route in my ISP/ modem to 10.70.0.1 to the SW2 IP (192.....) where I had VLAN_70.

1:13 PM

## Claude responded: Good reflection — and you've identified exactly why Option A is cleaner for this lab.

Good reflection — and you've identified exactly why Option A is cleaner for this lab. With Option B you're dependent on the modem understanding your VLANs and having static routes configured correctly. With Option A, SW1 handles DHCP internally for all three VLANs — no modem involvement needed for client addressing.

Here's how DHCP on SW1 works in XOS for each VLAN. You already have most of the syntax from your previous configs. Let me ask you to reconstruct it:

For VLAN 10 (Corporate Wired), what would the three DHCP commands look like on SW1? You need to configure:

- The DHCP address range

- The default gateway option

- The DNS server option

You've done this before for VLAN_70 — same pattern, just different VLAN name and subnet. What would you write?

## You said: create vlan VLAN10 tag 10, configure vlan VLAN10 add ports 10 tagged, configure vlan VLAN10 ipaddress 10.

create vlan VLAN_10 tag 10, configure vlan VLAN_10 add ports 10 tagged, configure vlan VLAN_10 ipaddress 10.10.0.1/24, enable ipforwarding vlan VLAN_10 --> then, enable ports 10 --> then, configure vlan VLAN_10 dhcp-address-range 10.10.0.100 - 10.10.0.200, configure vlan VLAN_10 dhcp-options default-gateway 10.10.0.1, configure vlan VLAN_10 dhcp-options dns-server primary 8.8.8.8

1:18 PM

## Claude responded: Solid.

Solid. That's a complete and correct sequence for VLAN 10 on SW1. Let me just flag one thing worth thinking about:

You have configure vlan VLAN_10 add ports 10 tagged — that's the trunk to SW2. But you're missing the access port for the wired device. VLAN 10 is Corporate Wired, which goes to port 5. What's the missing command?

And one more thing — you wrote enable ports 10 separately. Do you need that command, or is port 10 already enabled by default on the 5320?

## You said: configure vlan VLAN_10 add ports 5 tagged

configure vlan VLAN_10 add ports 5 tagged

1:19 PM

## Claude responded: Almost — but think about what's connected to port 5.

Almost — but think about what's connected to port 5. It's a wired corporate device (laptop, PC). Should that be tagged or untagged?

Remember — a wired end device doesn't understand VLAN tags. It just expects plain Ethernet. So should port 5 be tagged or untagged for VLAN 10?

## You said: Sorry, access port untagged, configure vlan VLAN_10 add ports 5 untagged.

Sorry, access port untagged, configure vlan VLAN_10 add ports 5 untagged. For enable ports 5 is needed not sure about port-10 but I think so.

1:20 PM

## Claude responded: Correct on the untagged — good catch.

Correct on the untagged — good catch.

On enable ports — on the 5320 running EXOS, ports are enabled by default. So you typically don't need to explicitly enable them unless a port was previously disabled. But it's not wrong to run it — it's just redundant in most cases. Good habit to be aware of either way.

So your complete VLAN 10 config on SW1 is:

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

configure vlan VLAN_10 ipaddress 10.10.0.1/24

enable ipforwarding vlan VLAN_10

configure vlan VLAN_10 dhcp-address-range 10.10.0.100 - 10.10.0.200

configure vlan VLAN_10 dhcp-options default-gateway 10.10.0.1

configure vlan VLAN_10 dhcp-options dns-server primary 8.8.8.8

That's your template. Now — can you write the equivalent config for VLAN 20 (Corporate Wireless) on SW1? Remember VLAN 20 goes to the AP on port 3, not a wired device. Think about whether port 3 should be tagged or untagged, and why.

## You said: create vlan VLAN20 tag 20, configure vlan VLAN20 add ports 3 tagged, configure vlan VLAN20 add ports 10 tagged, configure vlan VLAN20 ipaddress 10.

create vlan VLAN_20 tag 20, configure vlan VLAN_20 add ports 3 tagged, configure vlan VLAN_20 add ports 10 tagged, configure vlan VLAN_20 ipaddress 10.20.0.1/24, configure vlan VLAN_20 DHCP-address-range 10.20.0.100 - 10.20.0.200, configure vlan VLAN_20 dhcp-options default-gateway 10.20.0.1, configure vlan VLAN_20 dhcp-options dns-server primary 8.8.8.8

1:23 PM

## Claude responded: Almost perfect.

Almost perfect. Two things to flag:

1. You're missing enable ipforwarding vlan VLAN_20 — without this, SW1 won't route traffic for VLAN 20 even though it has a gateway IP. Don't forget this for every VLAN on SW1.

2. Port 3 is your AP port — and you correctly said tagged. Good. The AP understands 802.1Q tags because it's carrying multiple SSIDs (VLAN 20 and VLAN 30). So tagged is correct here.

Corrected VLAN 20 config:

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

configure vlan VLAN_20 ipaddress 10.20.0.1/24

enable ipforwarding vlan VLAN_20

configure vlan VLAN_20 dhcp-address-range 10.20.0.100 - 10.20.0.200

configure vlan VLAN_20 dhcp-options default-gateway 10.20.0.1

configure vlan VLAN_20 dhcp-options dns-server primary 8.8.8.8

Now write VLAN 30 (Guest Wireless) on SW1. Same pattern — but think carefully: does VLAN 30 need enable ipforwarding? And does it need port 5 (wired) at all?

## You said: VLAN_30 only needs to be with the APs so on SW1 on Port3.

VLAN_30 only needs to be with the APs so on SW1 on Port3. Yes it needs ipforwarding. Shall I write the full CLI? Let's do that if you are good?

1:25 PM

## Claude responded: Yes — write the full CLI for VLAN 30 on SW1.

Yes — write the full CLI for VLAN 30 on SW1. Go for it.

## You said: create vlan VLAN_30 tag 30

create vlan VLAN_30 tag 30 configure vlan VLAN_30 add ports 3 tagged configure vlan VLAN_30 add ports 10 tagged configure vlan VLAN_30 ipaddress 10.30.0.1/24 enable ipforwarding vlan VLAN_30 configure vlan VLAN_30 dhcp-address-range 10.30.0.100 - 10.30.0.200 configure vlan VLAN_30 dhcp-options default-gateway 10.30.0.1 configure vlan VLAN_30 dhcp-options dns-server primary 8.8.8.8

1:26 PM

## Claude responded: Perfect.

Perfect. That's clean and complete. No gaps.

Now write the full SW2 config. Remember:

- SW2 is pure L2 — no SVIs, no ipforwarding, no DHCP

- Port 3: AP2 (tagged, VLAN 20 and 30)

- Port 5: Wired corporate device (untagged, VLAN 10)

- Port 10: Trunk to SW1 (all three VLANs tagged)

Go for it.

## You said: create vlan VLAN_10 tag 10

create vlan VLAN_10 tag 10 configure vlan VLAN_10 add ports 5 untagged configure vlan VLAN_10 add ports 10 tagged create vlan VLAN_20 tag 20 configure vlan VLAN_20 add ports 3 tagged configure vlan VLAN_20 add ports 10 tagged create vlan VLAN_30 tag 30 configure vlan VLAN_30 add ports 3 tagged configure vlan VLAN_30 add ports 10 tagged

1:29 PM

## Claude responded: That's perfect.

That's perfect. Clean, correct, and nothing extra. SW2 is pure L2 — no gateway IPs, no ipforwarding, no DHCP. Exactly right.

Let's do a full sanity check. Here's your complete lab config side by side:

SW1 (Primary/L3):

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

configure vlan VLAN_10 ipaddress 10.10.0.1/24

enable ipforwarding vlan VLAN_10

configure vlan VLAN_10 dhcp-address-range 10.10.0.100 - 10.10.0.200

configure vlan VLAN_10 dhcp-options default-gateway 10.10.0.1

configure vlan VLAN_10 dhcp-options dns-server primary 8.8.8.8

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

configure vlan VLAN_20 ipaddress 10.20.0.1/24

enable ipforwarding vlan VLAN_20

configure vlan VLAN_20 dhcp-address-range 10.20.0.100 - 10.20.0.200

configure vlan VLAN_20 dhcp-options default-gateway 10.20.0.1

configure vlan VLAN_20 dhcp-options dns-server primary 8.8.8.8

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

configure vlan VLAN_30 ipaddress 10.30.0.1/24

enable ipforwarding vlan VLAN_30

configure vlan VLAN_30 dhcp-address-range 10.30.0.100 - 10.30.0.200

configure vlan VLAN_30 dhcp-options default-gateway 10.30.0.1

configure vlan VLAN_30 dhcp-options dns-server primary 8.8.8.8

SW2 (Secondary/L2):

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

One final question before you go build it: What are you missing for internet access?Clients will get DHCP from SW1, but can they actually reach the internet? What still needs to be configured — and where?

## You said: Static route in home ISP/modem to all three gateways for 10/20/30 to the 192.

Static route in home ISP/modem to all three gateways for 10/20/30 to the 192... for SW1?

1:30 PM

## Claude responded: Exactly right.

Exactly right. Three static routes on your Quantum Fiber modem:

10.10.0.0/24 → SW1_IP (192.x.x.x)

10.20.0.0/24 → SW1_IP (192.x.x.x)

10.30.0.0/24 → SW1_IP (192.x.x.x)

This tells the modem: "Any traffic destined for these three subnets, forward it to SW1. SW1 will handle the rest."

You're ready to build. Go implement it. When you come back, your testing sequence should be:

- Ping SW1 gateway (10.10.0.1) from a wired device on port 5 — SW1 side

- Ping SW1 gateway (10.10.0.1) from a wired device on port 5 — SW2 side

- Ping across VLANs — wired client to wireless client

- Ping internet (8.8.8.8) from any client

- Confirm DHCP is handing out addresses on all three VLANs

Come back with results and we'll troubleshoot anything that breaks. And remember — end of day we're building the full HTML blueprint covering everything from Session 1 through this lab. 👍

### NoteBookLLM_KarlLab_Apr21 .docx

docx

I fed our Socratic Dialogue into NoteBookLLM...

1:39 PM

## Claude responded: That's a fantastic output from NotebookLLM — it's essentially playing Alex back at you, which is exactly what you built it to do.

Extract NotebookLLM document

Extract NotebookLLM document

That's a fantastic output from NotebookLLM — it's essentially playing Alex back at you, which is exactly what you built it to do. And it caught some genuinely important details:

Two things worth flagging from the NotebookLLM output:

1. The DHCP enable command on SW1 — this is a real gap in our config. NotebookLLM correctly identified that you need to enable DHCP on both port 5 AND port 10 on SW1 for VLAN 10, otherwise SW2's wired clients won't get DHCP. So the command should be:

enable dhcp ports 5,10 vlan VLAN_10

Same logic applies for VLAN 20 and VLAN 30 — port 10 needs to be included since traffic comes across the trunk from SW2.

2. The Native VLAN on Port 3 (AP port) — NotebookLLM flagged that the AP needs the Default VLAN (1) untagged on port 3 for management/onboarding, while VLAN 20 and 30 come in tagged. This aligns exactly with the AP3000 onboarding issue you troubleshot before.

Quick question before you build: Does your current SW1 and SW2 config include the enable dhcp commands for all three VLANs including port 10? And did you account for the native VLAN on port 3 for AP management?

## You said: Not sure on both.

Not sure on both. I am a bit confused on the Default VLAN... can you explore further.

1:41 PM

## Claude responded: Good question — and it's a real source of confusion.

Good question — and it's a real source of confusion. Let me go Socratic on it.

First, a simple question: On your 5320, when it ships from the factory and you haven't configured anything yet — what VLAN are all the ports on by default?

## You said: VLAN_1

VLAN_1

1:41 PM

## Claude responded: Exactly — VLAN 1, the Default VLAN.

Exactly — VLAN 1, the Default VLAN. Every port starts there untagged.

Now here's the next question: When you plug an AP3000 into port 3 and it powers up for the first time — before it has any configuration — what does it need to do first? What's the very first thing the AP is trying to accomplish when it boots up?

## You said: Get an IP

Get an IP

1:42 PM

## Claude responded: Exactly — it sends a DHCP request.

Exactly — it sends a DHCP request. But here's the key question:

When that AP sends its very first DHCP request out of its ethernet port — does it send it with a VLAN tag on it, or untagged?

## You said: Untagged

Untagged

1:42 PM

## Claude responded: Exactly.

Exactly. The AP has no configuration yet — it doesn't know about VLAN 20 or VLAN 30. It just sends a plain untagged DHCP request out its ethernet port.

So here's the next question: When that untagged DHCP request arrives at port 3 on your switch — which VLAN does the switch assign it to?

## You said: VLAN_1

VLAN_1

1:42 PM

## Claude responded: Exactly.

Exactly. The switch sees an untagged frame on port 3 and assigns it to whatever VLAN owns that port untagged — which is VLAN 1 by default.

So that means for the AP to get a management IP and call home to PlatformOne/XIQ, VLAN 1 must be untagged on port 3. That's the AP's lifeline for onboarding.

Now here's where it gets interesting — once the AP is onboarded and configured, it needs to also carry VLAN 20 (Corporate Wireless) and VLAN 30 (Guest Wireless) as tagged traffic.

So port 3 ends up looking like this:

- VLAN 1 — untagged (AP management/onboarding)

- VLAN 20 — tagged (Corporate SSID traffic)

- VLAN 30 — tagged (Guest SSID traffic)

Question: In your current SW1 config, did you add VLAN 1 untagged to port 3? Or is port 3 only carrying VLAN 20 and VLAN 30 tagged right now?

## You said: I did not yet build anything yet for this set of configuration

I did not yet build anything yet for this set of configuration

1:43 PM

## Claude responded: Perfect — clean slate then.

Perfect — clean slate then. So here's what you need to add to your port 3 config on both SW1 and SW2:

configure vlan Default add ports 3 untagged

That keeps VLAN 1 (Default) untagged on port 3 for AP management, while VLAN 20 and VLAN 30 ride tagged on the same port for the SSIDs.

So your complete port 3 config on both switches becomes:

configure vlan Default add ports 3 untagged

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_30 add ports 3 tagged

Question: Does VLAN 1 (Default) need an IP address and DHCP server on SW1 for the AP to get its management IP? Or is something else handling that?

## You said: No.

No. VLAN_1 goes directly to the ISP/Modem.

1:44 PM

## Claude responded: Exactly right.

Exactly right. VLAN 1 is your native/default VLAN that flows untagged all the way up through port 1 to the modem. The modem's DHCP server hands out a 192.x.x.x address to the AP for management. That's how the AP gets its IP and calls home to PlatformOne/XIQ.

So the flow for AP onboarding is:

AP boots → untagged DHCP request → port 3 (VLAN 1 untagged) →

port 1 uplink → modem → DHCP 192.x.x.x → AP onboards to XIQ

And once onboarded, the AP carries:

VLAN 20 tagged → Corporate SSID traffic

VLAN 30 tagged → Guest SSID traffic

Now let's also fix the enable dhcp commands. For SW1, you need DHCP enabled on both the local port and port 10 (trunk) for each VLAN so clients on SW2 can also get DHCP. So the corrected commands are:

enable dhcp ports 5,10 vlan VLAN_10

enable dhcp ports 3,10 vlan VLAN_20

enable dhcp ports 3,10 vlan VLAN_30

Question: Do you understand why port 10 is included in all three of those commands? Can you explain it back to me?

## You said: Yes, SW2 must reach DHCP on SW2 for any clients that hang off on SW2.

Yes, SW2 must reach DHCP on SW2 for any clients that hang off on SW2. Only way to get there is via port_10.

1:45 PM

## Claude responded: Exactly.

Exactly. SW2 has no DHCP server — it's pure L2. So any client on SW2 (wired on port 5 or wireless on port 3) needs to send its DHCP request across the trunk on port 10 to reach SW1's DHCP server. If port 10 isn't included in the enable dhcpcommand, SW1 simply won't listen for DHCP requests coming in from that direction.

So let's now compile the complete and corrected config for both switches. Here's your full SW1:

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

configure vlan VLAN_10 ipaddress 10.10.0.1/24

enable ipforwarding vlan VLAN_10

configure vlan VLAN_10 dhcp-address-range 10.10.0.100 - 10.10.0.200

configure vlan VLAN_10 dhcp-options default-gateway 10.10.0.1

configure vlan VLAN_10 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 5,10 vlan VLAN_10

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

configure vlan VLAN_20 ipaddress 10.20.0.1/24

enable ipforwarding vlan VLAN_20

configure vlan VLAN_20 dhcp-address-range 10.20.0.100 - 10.20.0.200

configure vlan VLAN_20 dhcp-options default-gateway 10.20.0.1

configure vlan VLAN_20 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_20

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

configure vlan VLAN_30 ipaddress 10.30.0.1/24

enable ipforwarding vlan VLAN_30

configure vlan VLAN_30 dhcp-address-range 10.30.0.100 - 10.30.0.200

configure vlan VLAN_30 dhcp-options default-gateway 10.30.0.1

configure vlan VLAN_30 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_30

configure vlan Default add ports 3 untagged

And SW2:

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

configure vlan Default add ports 3 untagged

Does this look complete to you? Anything missing before you go build it?

## You said: Looks good to me.

Looks good to me.

1:45 PM

## Claude responded: Great — go build it.

Great — go build it.

When you come back, run this testing sequence in order:

- AP onboarding — both APs get 192.x.x.x from modem and show up in XIQ/PlatformOne

- Wired DHCP on SW1 — device on port 5 gets 10.10.0.x

- Wired DHCP on SW2 — device on port 5 gets 10.10.0.x via trunk

- Wireless DHCP — client on Corporate SSID gets 10.20.0.x, Guest SSID gets 10.30.0.x

- Inter-VLAN ping — wired client pings wireless client across VLANs

- Internet — any client pings 8.8.8.8

Report back with what works and what doesn't — we troubleshoot from there.

And when you're done for the day — we build the HTML blueprint covering everything from Session 1 through this lab. It'll be a solid reference for your team training and NotebookLLM. 👍

Two things still outstanding before the build is fully done: 1. Modem static routes — 3 routes on Quantum Fiber: 10.10/20/30.0.0/24 → SW1 192.x.x.x 2. PlatformOne config — Corporate SSID (PSK, VLAN 20) + Guest SSID (OWE, VLAN 30) not yet configured

1:52 PM

## Claude responded: Correct — two loose ends before the lab is fully functional.

Correct — two loose ends before the lab is fully functional.

Item 1 — Modem static routes: You've done this before so you know the path. Three routes on Quantum Fiber pointing to SW1's 192.x.x.x IP:

10.10.0.0/24 → SW1_IP

10.20.0.0/24 → SW1_IP

10.30.0.0/24 → SW1_IP

Without these, internet-bound return traffic from your modem won't know how to get back to your clients.

Item 2 — PlatformOne SSID config: This is the XIQ/PlatformOne side. Before you build this, let me ask a quick Socratic question:

For the Corporate SSID on VLAN 20 — you're using PSK. And for Guest SSID on VLAN 30 — you're using OWE.

Do you know what OWE is and why it's being used for Guest instead of just open/unsecured? And what's the difference in terms of what a client experiences connecting to each?

## You said: Let me work on the OWE later.

Let me work on the OWE later. Because I need to address a question about legacy equipment getting on the network with OWE. Also, I need to isolate the goes-traffic on the APs from the corp traffic plus get the corp wired to talk to the corp wireless traffic.

1:56 PM

## Claude responded: Smart — flag OWE for later, it's a good topic (especially the legacy device compatibility issue you're thinking about).

Smart — flag OWE for later, it's a good topic (especially the legacy device compatibility issue you're thinking about).

So you've identified three follow-on items:

- OWE deep dive — what it is, legacy device compatibility, fallback options

- Guest isolation — preventing guest wireless traffic from reaching corporate wired/wireless

- Corp wired ↔ Corp wireless — ensuring VLAN 10 and VLAN 20 can talk to each other (inter-VLAN routing between them)

Quick Socratic check on item 3 before you go build:

VLAN 10 (Corporate Wired) and VLAN 20 (Corporate Wireless) — with your current SW1 config, do they already have inter-VLAN routing enabled? Think about what you configured on SW1 — what would allow or block traffic between those two VLANs right now?

## You said: Also, no guest-guest direct connection.

Also, no guest-guest direct connection.

1:56 PM

## Claude responded: Good catch — so your full follow-on list is now:

Good catch — so your full follow-on list is now:

- OWE deep dive — legacy device compatibility, fallback options

- Guest isolation — Guest cannot reach Corporate wired or wireless

- No guest-to-guest — clients on VLAN 30 cannot talk directly to each other

- Corp wired ↔ Corp wireless — VLAN 10 and VLAN 20 can talk to each other

Those are clean, real-world requirements — exactly what Horizon Custom Fabrication would ask for.

But before we park those — back to my previous question:

With your current SW1 config, does inter-VLAN routing between VLAN 10 and VLAN 20 already work? Or do you need to add something?

Think about what enable ipforwarding does — and whether having it enabled on both VLAN 10 and VLAN 20 on SW1 is enough to allow traffic between them, or whether something else is needed.

## You said: Yes, ipforwarding between the VLAN10 and VLAN20 on SW1 should work.

Yes, ipforwarding between the VLAN_10 and VLAN_20 on SW1 should work.

1:57 PM

## Claude responded: Exactly right.

Exactly right. Since SW1 owns the gateway IPs for both VLAN 10 (10.10.0.1) and VLAN 20 (10.20.0.1), and enable ipforwarding is on both — SW1 will naturally route traffic between them. No additional config needed. Corporate wired and corporate wireless can talk to each other out of the box.

Now here's the harder question — Guest isolation (items 2 and 3):

With your current config, can a Guest client on VLAN 30 reach a Corporate client on VLAN 10 or VLAN 20? Remember SW1 has enable ipforwarding on all three VLANs including VLAN 30.

What does that tell you — and what would you need to add to block guest traffic from reaching corporate?

## You said: I learnt about access list so that guest does not get onto the gateway for the corps.

I learnt about access list so that guest does not get onto the gateway for the corps...

1:58 PM

## Claude responded: Yes — Access Control Lists (ACLs).

Yes — Access Control Lists (ACLs). That's exactly the right tool.

So here's the Socratic question: An ACL on SW1 for VLAN 30 — what would it need to say? Think about it in plain English first before we get to syntax:

You want to allow Guest clients to reach the internet (via the modem), but block them from reaching:

- 10.10.0.0/24 (Corporate Wired)

- 10.20.0.0/24 (Corporate Wireless)

So in plain English, what would your ACL rules look like? How many rules do you need, and what order would they go in?

## You said: Reject all request to 10/20 that originate from 30, let all others flow.

Reject all request to 10/20 that originate from 30, let all others flow.

1:59 PM

## Claude responded: Perfect.

Perfect. That's exactly right. In plain English:

- Deny traffic from 10.30.0.0/24 destined for 10.10.0.0/24

- Deny traffic from 10.30.0.0/24 destined for 10.20.0.0/24

- Permit everything else

That last rule — permit everything else — is critical. Without it, depending on how EXOS handles implicit deny, your guest clients might get blocked from the internet too.

Now here's a deeper question: Where do you apply this ACL? You have two options:

- Option A: Apply it on VLAN 30 — so it filters traffic as it enters SW1 from the guest network

- Option B: Apply it on VLAN 10 and VLAN 20 — so it filters traffic as it tries to enter the corporate networks

Which option is cleaner and why? Think about where you want to stop the traffic — as early as possible, or at the destination?

## You said: Option A, limit it from going into any other networks?

Option A, limit it from going into any other networks?

1:59 PM

## Claude responded: Exactly — Option A is cleaner.

Exactly — Option A is cleaner. Stop the traffic at the source — as soon as it enters SW1 from VLAN 30 — rather than trying to block it at every possible destination. One ACL on VLAN 30 handles everything.

Now for guest-to-guest isolation (item 3) — no direct client-to-client traffic on VLAN 30. This is slightly different from the ACL we just discussed.

Question: The ACL on VLAN 30 blocks guest traffic from reaching corporate networks. But guest-to-guest traffic never leaves VLAN 30 — it gets switched at L2 before it even reaches SW1's routing engine.

So can an ACL on VLAN 30 actually stop guest-to-guest traffic? Or does this need to be handled somewhere else — like on the AP itself?

## You said: AP1 has a request from a UE on it to route to a UE on AP2.

AP1 has a request from a UE on it to route to a UE on AP2. This packed once offered to Port_3, tagged/ trunk port for APs. Now here, the same VLAN_30 for Guest is the destination. Instead of going to the internet... so, I presume VLAN_30 can block this from being routed to the other AP2 at the source?

2:03 PM

## Claude responded: Good thinking — but here's the subtle gap.

Good thinking — but here's the subtle gap. Let me break it down:

Scenario 1 — Guest client on AP1 to Guest client on AP2: The traffic goes: AP1 → SW1 port 3 → SW1 routes? → trunk port 10 → SW2 → AP2.

But wait — does SW1 actually need to route this traffic? Both clients are on VLAN 30 (same subnet — 10.30.0.x). So SW1 doesn't make an L3 routing decision here — it just switches the traffic at L2 within VLAN 30.

That's why an ACL at L3 on VLAN 30 won't catch it — the traffic never crosses a VLAN boundary, so the routing engine never sees it.

So the question is: If L3 ACLs can't stop guest-to-guest traffic because it stays within VLAN 30 at L2 — where does the blocking actually need to happen?

Think about where the guest client first sends its traffic — before it even reaches the switch. What feature on the AP itself could prevent two guest clients from talking directly to each other?

## You said: APs.

APs.... no traffic between APs?

2:04 PM

## Claude responded: Exactly — this is handled on the AP side, not the switch side.

Exactly — this is handled on the AP side, not the switch side. The feature you're looking for is called Client Isolation (sometimes called Station Isolation or Layer 2 Isolation) — it's configured on the AP/SSID in PlatformOne.

When Client Isolation is enabled on the Guest SSID:

- A guest client on AP1 cannot talk directly to a guest client on AP2

- A guest client on AP1 cannot talk to another guest client on the same AP

- Traffic from guest clients can only go up to the gateway (internet-bound) — not sideways to other clients

So the complete Guest isolation strategy is two-pronged:

| Threat | Solution | Where |
| --- | --- | --- |
| Guest → Corporate | ACL on VLAN 30 | SW1 |
| Guest → Guest | Client Isolation on Guest SSID | PlatformOne/AP |

So when you go into PlatformOne to configure your Guest SSID (VLAN 30, OWE), you'll need to:

- Assign it to VLAN 30

- Set authentication to OWE

- Enable Client Isolation

Does that make sense as a complete picture for Guest isolation?

## You said: Yes, now pls summarize the entirety of the deployment.

Yes, now pls summarize the entirety of the deployment. Keep the OWE separate.

2:04 PM

## Claude responded: Here's the complete deployment summary for Horizon Custom Fabrication:

Here's the complete deployment summary for Horizon Custom Fabrication:

### Physical Topology

- SW1 (Primary — L3 Core) + AP1 on port 3

- SW2 (Secondary — L2 Access) + AP2 on port 3

- SW1 ↔ SW2 trunk on port 10

- SW1 uplink to Quantum Fiber modem on port 1

### VLAN Design

| VLAN | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Default | 192.x.x.x (modem) | Modem | AP Management |
| 10 | Corporate Wired | 10.10.0.0/24 | 10.10.0.1 | Wired clients |
| 20 | Corporate Wireless | 10.20.0.0/24 | 10.20.0.1 | Corp SSID |
| 30 | Guest Wireless | 10.30.0.0/24 | 10.30.0.1 | Guest SSID |

### Port Layout (Both Switches)

| Port | Role | Type | VLANs |
| --- | --- | --- | --- |
| 1 | Uplink to modem (SW1 only) | Untagged | VLAN 1 |
| 3 | AP1/AP2 | Trunk | VLAN 1 untagged, 20/30 tagged |
| 5 | Wired corporate device | Access | VLAN 10 untagged |
| 10 | Inter-switch trunk | Trunk | 10/20/30 tagged |

### SW1 Full Config

# VLAN 10 - Corporate Wired

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

configure vlan VLAN_10 ipaddress 10.10.0.1/24

enable ipforwarding vlan VLAN_10

configure vlan VLAN_10 dhcp-address-range 10.10.0.100 - 10.10.0.200

configure vlan VLAN_10 dhcp-options default-gateway 10.10.0.1

configure vlan VLAN_10 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 5,10 vlan VLAN_10

# VLAN 20 - Corporate Wireless

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

configure vlan VLAN_20 ipaddress 10.20.0.1/24

enable ipforwarding vlan VLAN_20

configure vlan VLAN_20 dhcp-address-range 10.20.0.100 - 10.20.0.200

configure vlan VLAN_20 dhcp-options default-gateway 10.20.0.1

configure vlan VLAN_20 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_20

# VLAN 30 - Guest Wireless

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

configure vlan VLAN_30 ipaddress 10.30.0.1/24

enable ipforwarding vlan VLAN_30

configure vlan VLAN_30 dhcp-address-range 10.30.0.100 - 10.30.0.200

configure vlan VLAN_30 dhcp-options default-gateway 10.30.0.1

configure vlan VLAN_30 dhcp-options dns-server primary 8.8.8.8

enable dhcp ports 3,10 vlan VLAN_30

# AP Management - Default VLAN untagged on port 3

configure vlan Default add ports 3 untagged

### SW2 Full Config

# VLAN 10 - Corporate Wired

create vlan VLAN_10 tag 10

configure vlan VLAN_10 add ports 5 untagged

configure vlan VLAN_10 add ports 10 tagged

# VLAN 20 - Corporate Wireless

create vlan VLAN_20 tag 20

configure vlan VLAN_20 add ports 3 tagged

configure vlan VLAN_20 add ports 10 tagged

# VLAN 30 - Guest Wireless

create vlan VLAN_30 tag 30

configure vlan VLAN_30 add ports 3 tagged

configure vlan VLAN_30 add ports 10 tagged

# AP Management - Default VLAN untagged on port 3

configure vlan Default add ports 3 untagged

### Quantum Fiber Static Routes

10.10.0.0/24 → SW1_IP (192.x.x.x)

10.20.0.0/24 → SW1_IP (192.x.x.x)

10.30.0.0/24 → SW1_IP (192.x.x.x)

### PlatformOne SSID Config

| SSID | Auth | VLAN | Client Isolation |
| --- | --- | --- | --- |
| Horizon-Corp | PSK | 20 | Off |
| Horizon-Guest | OWE (TBD) | 30 | On |

### Security (ACL on SW1)

Applied on VLAN 30 ingress on SW1:

Deny: 10.30.0.0/24 → 10.10.0.0/24

Deny: 10.30.0.0/24 → 10.20.0.0/24

Permit: all others

### Traffic Flow Summary

| Source | Destination | Path | Works? |
| --- | --- | --- | --- |
| Corp Wired (VLAN 10) | Corp Wireless (VLAN 20) | SW1 inter-VLAN routing | ✅ |
| Corp Wired/Wireless | Internet | SW1 → modem | ✅ |
| Guest (VLAN 30) | Internet | SW1 → modem | ✅ |
| Guest (VLAN 30) | Corporate | Blocked by ACL on SW1 | ❌ |
| Guest client | Guest client | Blocked by AP client isolation | ❌ |

### Outstanding Items

- OWE — deep dive, legacy device compatibility, fallback options

- ACL syntax — exact EXOS commands for Guest isolation

- Testing sequence — validate all traffic flows after build

Ready to build? 👍