See, Claude_NewLab_Karl_Dialogue_Apr21.docs

Hey! I've been going through the dialogue in Claude_NewLab_Karl_Dialogue_Apr21 and it’s like looking at a master plan for the Horizon Custom Fabrication project. As a fellow student, I’m really starting to see how our "independent island" experiments are evolving into a professional Primary/Secondary (L3 Core/L2 Access) architecture.

Here is my "Alex-style" review of the big technical leaps we’re making in this new lab:

1. The "Option A" Breakthrough

In our earlier labs, we were using "Option B," where we were basically at the mercy of the home modem for everything. In this new plan, we’re moving to Option A, where Switch 1 (Primary) handles DHCP internally for all three VLANs (10, 20, and 30). This is huge because it means the modem doesn't need to understand our internal VLAN structure anymore; it just needs to know where to send return traffic.

2. The Port Discipline

The port layout in the dialogue is really clean now:

- Port 1: The untagged uplink to the modem on VLAN 1.

- Port 3: An 802.1Q Trunk for the APs because they have to carry both Corporate (VLAN 20) and Guest (VLAN 30) SSIDs.

- Port 5: A simple Access Port on VLAN 10 for wired corporate devices.

- Port 10: The critical Inter-Switch Trunk that "stretches" all three VLANs across both switches.

3. The "Engine" (Layer 3) Logic

The dialogue reinforces that "SVI Ownership" is the key to being a Core switch.

- Switch 1 is the only one that gets IP addresses for the VLANs (10.10.0.1, 10.20.0.1, 10.30.0.1).

- Switch 2 stays "quiet" at Layer 3; it has no IPs on these data VLANs and acts as a pure Layer 2 pipe.

- The Golden Rule: We can’t forget enable ipforwarding on every single VLAN on Switch 1, or our clients will be stranded with an IP but no way to talk to the next hop.

4. Final Mission Success (The Modem Handoff)

I noticed the dialogue ends with the most critical part of the "Packet Journey": the modem static routes. Since our Quantum Fiber modem only knows about the 192.168.0.x network, we have to tell it manually that to find 10.10.0.0, 10.20.0.0, or 10.30.0.0, it has to talk to Switch 1’s IP address.

One quick "student gap" check for me: In the configuration for Switch 2, you have Port 5 set as an access port for VLAN 10. If I plug my laptop into Port 5 on Switch 2, and the DHCP server is all the way over on Switch 1, how does the DHCP request get there if there is no IP address on Switch 2 to guide it? (I think the answer has to do with that Port 10 Trunk, right?)

What is your goal for our next step in this build?

Yes, the trunk_10 reaches out to SW1 which provide DHCP to Port_5 wired UE.

That is exactly right! As we’ve been mapping out in our "New Rosetta Stone" logic, that Port 10 Trunk is the critical bridge that lets Switch 1 act as the "brain" for the entire lab.

Based on our architectural review and your recent lab notes, here is why that specific path works and the one "Alex-style" detail we can't forget:

1. The Packet Journey across the Trunk

When you plug your Wired UE into Port 5 on Switch 2, the DHCP request follows this path:

- Ingress: The request enters Switch 2 on Port 5 (Untagged).

- Tagging: Switch 2 identifies it as VLAN 10 (Corporate Wired) and adds the 802.1Q tag.

- Transport: The tagged frame travels across the Port 10 Trunk to Switch 1.

- Service: Switch 1 receives the frame and passes it to the internal DHCP Server engine because Switch 1 owns the SVI (10.10.0.1).

2. The Critical "Port 10" Command

One thing I noticed in our April 7 lessons is that EXOS is extremely literal about where it listens for DHCP requests. For Switch 1 to "hear" the request coming from Switch 2, we have to make sure the trunk port itself is enabled for DHCP.

On Switch 1, the command must include the trunk port: enable dhcp ports 5,10 vlan VLAN_10

If we only enabled it on Port 5, Switch 1 would only provide IPs to devices plugged directly into itself, and the UE on Switch 2 would stay in a "Searching..." state.

3. Verification Check

To prove this is working, I usually run show dhcp-server vlan VLAN_10 on Switch 1.

- You should see the MAC address of your Switch 2 UE in the "Assigned" list.

- The "Lease Time Left" will confirm that the "handshake" between the two switches across the trunk was successful.

Alex's next question for our build: Now that the trunk is successfully passing DHCP traffic to your wired UE, are you ready to test the Inter-VLAN routing? If you plug a second device into Switch 1, can that device ping the UE on Switch 2 even though they are on different physical boxes?

No I have not built yet. Will do so now.

Awesome! I'm standing by. As a fellow student who's been through those "black hole" sessions with you, I’m excited to see this Primary/Secondary architecture come to life.

Before you start plugging in and pasting, I’ve got three "Alex-style" pro-tips from our notes to keep in the back of your mind while you're in the zone:

- The "Karl" Rule: Don't forget that enable ipforwarding vlan Default on Switch 1. If you miss that one, your clients will get an IP address and look perfectly fine, but they'll be "ghosted" by the internet because the return traffic won't know how to cross back from the modem's subnet to yours.

- Mind the Gap (Spaces in DHCP): When you're setting up the DHCP ranges for VLANs 10, 20, and 30, remember EXOS is super picky—make sure there are spaces on both sides of the dash (e.g., 10.10.0.100 - 10.10.0.200) or the switch will throw a syntax error.

- The Native VLAN Handshake: On Port 3, keep the Default VLAN (1) untagged for the AP3000 to "call home" and get its management IP, while your SSIDs for the Corporate and Guest networks come in tagged.

Look at new document w/ Claude calibration for my work with your inputs... Claude_NewLab_Karl_Dialogue_Apr21_v2.docx

Hey! I just went through Claude_NewLab_Karl_Dialogue_Apr21_v2 and as your student-persona Alex, I’m really impressed by how much we tightened up the logic in this "v2" version. We’ve moved from basic connectivity to a much more professional Primary/Secondary (L3 Core/L2 Access) design for "Horizon Custom Fabrication".

Here is my review of the major refinements and the "lessons learned" we've now baked into the plan:

1. The "DHCP Reach-Back" Fix (Port 10)

One of the biggest catches in this new doc is the enable dhcp command on Switch 1.

- The Refinement: In v1, we only enabled DHCP on the local ports. You correctly identified that for a wired device on Switch 2 (Port 5) to get an IP, Switch 1 has to be listening for DHCP requests coming across the Port 10 Trunk.

- The Command: enable dhcp ports 5,10 vlan VLAN_10. Without adding port 10 there, our Switch 2 users would be stuck with "self-assigned" IPs.

2. Solving the AP Onboarding Dilemma (Port 3)

We finally cleared up the confusion around the Default VLAN and the APs.

- The Logic: You correctly mapped out that an AP3000 starts "dumb"—it has no SSID or VLAN config at boot, so it sends an untagged DHCP request.

- The Solution: We must keep VLAN 1 (Default) as untagged on Port 3 so the AP can talk to the modem and get its 192.x.x.x management IP to call home to XIQ. Once it's "online," it then starts handling the tagged traffic for VLAN 20 and 30.

3. IP Design and Gateway Ownership

The addressing scheme is now very clean:

- VLAN 10 (Wired): 10.10.0.1

- VLAN 20 (Corp): 10.20.0.1

- VLAN 30 (Guest): 10.30.0.1.

- The Handoff: Only Switch 1 owns these IPs (the SVIs). Switch 2 is just a "pipe" that passes the traffic up the trunk. This follows the Option A logic where Switch 1 is the authoritative DHCP server and router for the whole lab.

4. The "Karl Rule" (Internet Return Path)

We didn't forget the most important lesson from our April 7 session: the Quantum Fiber modem static routes.

- The Configuration: Since the modem doesn't know about our 10.x.x.x subnets, we have to manually tell it: "To find the 10.10.0.0, 10.20.0.0, or 10.30.0.0 networks, send the packets to Switch 1’s IP (192.168.0.x)".

--------------------------------------------------------------------------------

Alex's Socratic "Gap Check" for the Final Build:

I feel 100% confident in the VLAN and DHCP logic now. But before you start pasting this into the consoles, I have one question to make sure we don't accidentally knock ourselves out of the switches:

If we are using Port 1 as our management uplink to the modem (VLAN 1), and we are about to plug a cable into Port 10 to connect the two switches together (carrying VLAN 10, 20, and 30), what happens if we accidentally add "VLAN 1" to that Port 10 trunk as well? (Hint: Both switches are already connected to each other through the modem on VLAN 1).

How do you plan to verify the inter-switch trunk is actually active before you start the wireless tests?

I have been musing that too. If Port_10 has VLAN_1 then traffic would pass through and ping-pong? Not sure. Inter-switch trunk tests could be to trace a ping from an UE on AP1 to a UE on AP2 or vice versa?

That is a really sharp catch! As your fellow student Alex, I’ve been looking at the Layer 2 loop prevention sections in the user guides, and your "ping-pong" concern is spot on. If we aren't careful with VLAN 1, we could accidentally turn our lab into a "broadcast storm" generator.

Here is the breakdown of why that happens and how we should test our trunk:

1. The VLAN 1 "Triangle" (The Loop)

You are exactly right about the danger of adding VLAN 1 to Port 10.

- The Path: Right now, both of your switches are connected to the Quantum Fiber modem on Port 1 using VLAN 1 (Default). This means there is already a Layer 2 path between the switches through the modem.

- The Conflict: If you also add VLAN 1 to the Port 10 Trunk, you create a physical triangle: SW1 ↔ Modem ↔ SW2 ↔ SW1.

- The Result: Without Spanning Tree (STP) to block one of those paths, a single broadcast frame would loop around that triangle indefinitely—that's the "ping-ponging" you mentioned.

- ** Alex's Tip:** In EXOS, the Default VLAN is usually assigned to STPD s0 (Spanning Tree Domain 0) by default to prevent this exact nightmare.

2. Inter-Switch Trunk Verification

Your idea to trace a ping between UEs on different APs is the absolute "Gold Standard" test for our Primary/Secondary architecture.

If you have a UE on AP1 (Switch 1) try to reach a UE on AP2 (Switch 2), the packet has to complete a "Triple Jump":

- L3 Gateway Jump: The packet must hit its gateway IP on Switch 1 (Primary).

- The Routing Decision: Switch 1 realizes the destination UE is on a different subnet (e.g., jumping from VLAN 40 to VLAN 70).

- The Trunk Transit: Switch 1 rewrites the frame and sends it across Port 10 to Switch 2.

How to "See" the Trunk Working (Alex's Checklist)

Instead of just guessing if it's working, we can use these "truth" commands:

- Check the MAC Table: Run show fdb port 10 on Switch 1. If you see the MAC address of the iPhone that is physically connected to Switch 2, you know the trunk is successfully "learning" across the wire.

- Verify the Gateway: From the Switch 1 console, try ping vr VR-Default 10.70.0.x (the UE on Switch 2). This confirms Switch 1 can "reach back" through the trunk to the edge.

- Trace the Path: If you have a laptop, you can run a traceroute. You should see the first hop as your Primary Switch gateway (10.x.0.1) and the next hop as the destination UE.

My Socratic check for you: Since our AP3000s are also connected via VLAN 1 for management, what command would you run to make sure the AP on Switch 2 is actually showing up as a "neighbor" to Switch 1 through the modem link? (Hint: it's a neighbor discovery protocol common to both Cisco and Extreme).

What is the first pair of devices you're going to use for the UE-to-UE test?

LLDP?

Hey! As your fellow student, I’ve been digging into the LLDP (Link Layer Discovery Protocol) section of the manuals. It’s basically the "universal language" that lets different pieces of hardware—like our 5320 switches and the AP3000s—introduce themselves to each other.

In our lab, LLDP isn't just a "nice-to-have" tool; it’s actually the engine for our automation. Here is the breakdown of why LLDP is critical for our current build:

1. The "AP Handshake" (Fabric Attach)

In our Primary/Secondary architecture, LLDP is the carrier for Fabric Attach (FA).

- When you plug the AP3000 into Port 3, it uses LLDP to "advertise" its presence.

- The switch receives these LLDP packets and realizes, "Oh, this is an AP, and it needs VLAN 20 and 30".

- Without LLDP enabled, that "Option A" automation we were talking about won't work, and the AP will stay in a "disconnected" state.

2. LLDP Rosetta Stone (Vendor Mapping)

Since we’re building a multi-vendor mapping guide, here is how the "discovery" command translates across your different platforms:

| Feature | Extreme (EXOS) | Extreme (VOSS) | Cisco IOS-XE |
| --- | --- | --- | --- |
| Enable Global | Enabled by default | lldp tx-interval <sec> | lldp run |
| Enable Port | enable lldp ports 3,10 | lldp status 1/3 txAndrx | lldp transmit/receive |
| Verification | show lldp neighbors | show lldp neighbor | show lldp neighbors |

3. VoIP and Priority (LLDP-MED)

You mentioned wanting to set up QoS and prioritize corporate traffic. This is where LLDP-MED (Media Endpoint Discovery) comes in.

- It allows the switch to automatically tell a device (like a VoIP phone) which Voice VLAN and DSCP priority to use.

- On EXOS, we can use the command: configure lldp port all advertise vendor-specific med policy application voice vlan <vlan_name> dscp 46.

4. Mandatory vs. Optional Info (TLVs)

LLDP uses TLVs (Type-Length-Values) to share data.

- Mandatory: Every packet must share the Chassis ID (the switch MAC) and the Port ID.

- Alex's Tip: In our Port 10 Trunk test, if you run show lldp neighbors detailed, you can actually see the Management IP of the other switch, which is a great way to verify Layer 2 connectivity even if pings aren't working yet.

Alex's Socratic Check for the Build:

Since we're using Port 3 for the APs and Port 10 for the trunk, what is the first show command you're going to run once the cables are in to prove that Switch 1 and Switch 2 can actually "see" each other's identities?

And do you want me to write the specific LLDP-MED command block for our Corporate VLAN priority once we move into the QoS phase?

Read the new file... Claude_NewLab_Karl_Dialogue_Apr21_v3.docx for updates

Hey! I see that v3 of our dialogue is now in the notebook, but it looks like it is currently excluded from my view (it's marked as "excluded by user" in my current session notes). I can’t "read" those specific updates yet because I don't have access to the text in that file.

Once you check the box to include Claude_NewLab_Karl_Dialogue_Apr21_v3.docx in our current session, I’ll be able to see the specific refinements you and Claude made!

In the meantime, I’ve been looking at the Network Requirements document and the EXOS User Guide to get a head start on the two big upgrades you mentioned earlier:

1. The QoS Priority Plan

From the sources, I see that we can use QoS Profiles (QP1–QP8) to give your Corporate Wired (VLAN 10) and Wireless (VLAN 20) traffic "Priority Access".

- The Logic: We can map those VLANs to a higher profile like QP5 or QP6 so they get serviced before the Guest (VLAN 30) traffic.

- Verification: We can run show ports <port_list> qosprotocol to see if the traffic is actually hitting the "Express" queues.

2. The Isolation / "Micro-Segmentation" Plan

You want to make sure users on VLAN 40 and 70 can't talk to each other even if they are on the same segment. The sources suggest two ways to handle this in EXOS:

- Private VLANs (PVLANs): We can configure those VLANs as "Isolated Subscriber VLANs". This forces every client to talk only to the gateway and blocks peer-to-peer traffic.

- Policy-Based ACLs: We can create a deny rule for the specific roles to prevent lateral movement.

Once you've enabled that v3 file, shall we start by writing the specific CLI blocks for the Private VLAN isolation, or should we focus on the QoS mapping for your Corporate data first?

See now?

Yes, I see it clearly now! I’ve reviewed Claude_NewLab_Karl_Dialogue_Apr21_v3.docx, and as your fellow student Alex, I can see we’ve really leveled up the "Horizon Custom Fabrication" blueprint.

This version moves beyond basic connectivity and builds in professional-grade security and "reach-back" logic. Here is my "student's-eye view" of the most critical updates I found in the v3 doc:

1. The DHCP "Reach-Back" Fix (Port 10)

We caught a big one here. For a wired user on Switch 2 (Port 5) to get an IP address, Switch 1 (the Primary) needs to be listening for DHCP requests coming across the Port 10 Trunk.

- The Update: We revised the Switch 1 command to enable dhcp ports 5,10 vlan VLAN_10. Without adding Port 10, Switch 1 would "ignore" any devices on Switch 2 asking for an identity.

2. The AP Management Solution (Port 3)

We finally solved the AP onboarding puzzle. Since the AP3000 starts "dumb" at boot, it needs an untagged path to find the modem and XIQ.

- The Strategy: We are keeping VLAN 1 (Default) as untagged on Port 3 so the AP gets its management IP (192.x.x.x) from the modem.

- The Data Path: We then layer VLAN 20 and 30 as tagged on the same port to carry the actual SSID traffic once the AP is online.

3. The Two-Pronged Guest Isolation

You had a great catch about guest-to-guest traffic. Since that traffic stays inside the same VLAN, an ACL on the switch won't stop it. Our v3 strategy is now:

- Guest → Corporate (North-South): Blocked by an ACL on Switch 1 (Deny 10.30 to 10.10/20, Permit all others).

- Guest → Guest (East-West): Blocked by Client Isolation configured in the PlatformOne SSID settings.

4. "Default" Inter-VLAN Routing

I also saw we confirmed that since Switch 1 owns the gateway IPs for both VLAN 10 and 20, and ipforwarding is enabled, Corporate Wired and Wireless will talk to each other automatically without any extra static routes between the switches.

The Alex "Safety Check" before the Build:

I’ve got the full summary from v3 locked in. Before you start the physical wiring, I have one question for our Layer 1 setup:

Since we are using Port 1 as our "Management Path" to the modem (VLAN 1), and we are turning Port 10 into our "Data Path" for VLANs 10, 20, and 30, are we 100% sure we are leaving VLAN 1 OFF of that Port 10 Trunk? (If we add it to both, we might accidentally create that "loop triangle" through the modem we talked about earlier!)

I’m ready for your results! Which switch are you going to console into first?

To be 100% sure VLAN1 is off on Port10 Trunk we could use a command to wipe it off if there or look at the config?

That is a great question! As your fellow student Alex, I’ve been looking at the "loop triangle" risk we talked about (SW1 ↔ Modem ↔ SW2 ↔ SW1), and you're right—being "100% sure" is the only way to avoid a broadcast storm.

To handle this in EXOS, we should definitely do both: use the "wipe" command to be safe and then "look at the config" to verify.

1. The "Wipe" Command (Administrative Removal)

In EXOS, the way to ensure a port is completely removed from a specific VLAN is using the delete ports verb. Since all ports on a factory-reset switch are usually untagged members of the Default VLAN by default, you should run this on both switches:

configure vlan Default delete ports 10

Note: In EXOS, you don't "no switchport trunk allowed vlan 1" like in Cisco; you explicitly tell the VLAN to "delete" that port from its membership list.

2. The "Look at Config" Command (Verification)

Once you've run the delete command, you can use the "truth commands" to verify Port 10 is no longer a member of the Default VLAN.

- show vlan Default: This will show you the specific member list for VLAN 1. You want to make sure "10" does not appear in the Untag or Tag columns.

- show vlan detail: This is the deep-dive version. It will list every port and its tagging status (Untagged, Tagged, or None) for every VLAN in the lab.

- show ports 10 vlan: This is a great "shortcut" command that looks at the world from the perspective of the port itself. It will list every VLAN that Port 10 is allowed to carry.

Alex’s Socratic "Safety Check":

We know Port 10 is now clean of VLAN 1, and it's carrying our data VLANs (10, 20, 30) as tagged.

But remember our "Karl Lesson" from April 7: Switch 1 is our primary router. If a packet from a client on Switch 2 comes across that Port 10 trunk and needs to go to the internet, it has to "jump" from your data VLANs to the Default VLAN on Port 1 to reach the modem.

What is the one command we must run on Switch 1 to make sure the internal "engine" allows traffic to cross that boundary from VLAN 10/20/30 over to the Default VLAN? (Hint: It’s the command that turns the switch from a Layer 2 bridge into a Layer 3 router).

Shall I provide the specific show fdb commands to run once you plug the cable into Port 10 to prove the trunk is actually "learning" MAC addresses?