I will give you my full day's work here... until QoS and claissificaiton, from Claude... review pls and figure out what I need to do to solve
the probelme to assign QoS on Switches and APs for isolation for guest... /Users/khukhan/Downloads/Claude_FullDay_Apr21_Until\ QoS\ _\
Classification.docx

Hey! As your fellow student Alex, I’ve reviewed your progress through the v3 calibration and the new Network Requirements. You have successfully moved the lab from "independent islands" to a professional Primary (L3 Core) / Secondary (L2 Access) architecture.

To solve the remaining problem of assigning QoS for priority access and finalizing Guest Isolation, we need to implement a three-part strategy across your Extreme 5320 switches and the PlatformOne AP policy.

1. QoS: Creating "Priority Lanes" for Corporate Traffic

You want Corporate Wired (VLAN 10) and Corporate Wireless (VLAN 20) to have priority over Guest traffic. In EXOS, this is handled by mapping traffic groups to QoS Profiles (QP1–QP8).

- The Strategy: By default, everything sits in QP1 (the "Slow Lane"). We will move your Corporate VLANs to QP5 (a "Medium-High Lane").

- Switch 1 CLI Commands:

- Switch 2 Consideration: Since SW2 is pure Layer 2, it will automatically respect the 802.1p CoS (Class of Service) tags inside the frames as they travel across the Port 10 Trunk, provided you enable examination: enable dot1p examination ports 3,10.

2. AP-Level QoS (PlatformOne)

To ensure wireless traffic is prioritized the moment it hits the air, you must configure the SSID markings in ExtremeCloud IQ.

- The Action: In your Horizon-Corp SSID settings, set the QoS marking to CoS 5.

- The Result: When an iPhone sends a packet to AP1, the AP wraps it in a header marked with "Priority 5." When Switch 1 receives it on Port 3, it sees that marking and immediately puts it into the QP5 hardware queue.

3. Finalizing Guest Isolation (The Two-Pronged Approach)

Your v3 blueprint correctly identifies that Guest isolation requires two different tools because of how traffic flows.

A. Guest → Corporate (North-South Isolation) Since this traffic tries to cross a VLAN boundary, Switch 1 (the router) must block it using an ACL.

- The Command (Switch 1):

- Note: Because this is a "deny" rule, EXOS handles the "permit all others" (to the internet) automatically if there are no other rules, but it is best practice to verify internet pings after applying.

B. Guest → Guest (East-West Isolation) As we discussed, this traffic never hits the switch's routing engine, so an ACL won't see it.

- The Action: In PlatformOne, enable Client Isolation on the Horizon-Guest SSID.

- The Result: AP1 and AP2 will physically prevent one guest client from "seeing" another guest client, even if they are on the same SSID.

Alex's Build Review Summary

- SW1 (Primary): Handles all SVIs, DHCP, Routing, and the Guest ACL.

- SW2 (Secondary): Handles L2 access and passes everything up the Port 10 Trunk.

- APs: Handle SSID-to-VLAN mapping and Client Isolation.

Next Step for you: Do you want to write the specific show commands we'll use to verify that a Corporate iPhone is actually landing in QP5 while a Guest iPhone stays in QP1?

No, see Claude_FullDay_Apr21 file... pls just uploaded and STUDY FULLY

Hey! As your fellow student Alex, I’ve gone through the Full Day Log (Apr 21) from start to finish. I see exactly where we hit the "exhaustion point" at 5:06 PM after that marathon session.

You did some incredible heavy lifting—moving from "independent islands" to a fully functioning Primary/Secondary (L3/L2) architecture for Horizon Custom Fabrication. I’ve locked in the "Big Wins" and the "Final Boss" hurdles we still have to clear.

The "State of the Lab" Recap (What's Working)

Based on the full study of your log:

- Layer 1/2 Infrastructure: SW1 and SW2 are linked via the Port 10 Trunk (VLANs 10, 20, 30 tagged). VLAN 1 is confirmed OFF that trunk to prevent loops.

- The "Karl Rule" Success: You successfully enabled ipforwarding vlan Default on SW1, which was the "magic key" that finally gave your clients internet access.

- DHCP Mastery: SW1 is successfully acting as the authoritative DHCP server for all segments, including providing IPs to clients on SW2 via the trunk.

- Licensing Realization: We confirmed the switches only have Standard Licenses, so Option A (NAT on the switch) is off the table. We are sticking with Option B (Quantum Fiber modem handles NAT).

--------------------------------------------------------------------------------

The Remaining Problem: QoS and Guest Isolation

We were right in the middle of this when we called it a day. You were very clear that you want the Switch to Enforce the priority, but you also wanted to understand how the APs handle it.

Here is the "Alex-style" study notes on how we solve this, avoiding the syntax errors we hit earlier:

1. QoS: The "Express Lane" (Switch Side)

Since you want the switch to enforce this, we use QoS Profiles. We decided on QP6 for Corporate to leave room for future control traffic in QP7/8.

- The Logic: We map the entire VLAN to the hardware queue.

- The Verified EXOS Command (SW1): configure vlan VLAN_10_Corporate_Wired qosprofile qp6 configure vlan VLAN_20_Corporate_Wireless qosprofile qp6 configure vlan VLAN_30_Guest_Wireless qosprofile qp1

2. QoS: The "Airtime" (AP Side)

The AP prioritizes traffic using WMM (Wi-Fi Multimedia).

- The Logic: In XIQ, we set the SSID to map to a specific "Access Category" (Voice, Video, Best Effort, Background).

- The Goal: Corporate SSID traffic gets tagged with CoS 5 or 6 the moment it enters the switch port, so the switch knows to put it in QP6 immediately.

3. Guest Isolation: The Two-Pronged Strategy

This is where the syntax got messy. We need to block Guests from the Corporate network, but NOT from the internet.

- North-South (Guest → Corp): This requires a Policy-Based ACL on SW1 because it's a routing decision. We found that the simple create access-list command failed because of "invalid attributes". The fix is using the Policy Editor (edit policy).

- East-West (Guest → Guest): We confirmed this must be done in PlatformOne (XIQ) using the Client Isolation toggle on the SSID, because that traffic never leaves the AP's radio to hit the switch.

Alex's Study Question: Now that we've reviewed the "Standard License" limit and the "Karl Rule" fix, are you ready to try the edit policy method for the Guest ACL one more time, or should we focus on the XIQ SSID markings for QoS first to get the wireless priority sorted?

I just want to enable QoS so that corporate traffic gets higher priority than guest. Also, I want to isolate that guests ONLY have access to the internet and NOT to any corporate end user or any other guest. I want to enable these in XIQ and CLI both if possible. Or with XIQ for APs and CLI for Switches? Whichever makes sense

Hey! As your fellow student Alex, I’ve been studying the "Final Boss" hurdles of our build. Since we are using a Primary/Secondary (L3/L2) architecture, we can divide the work exactly how you suggested: XIQ for the APs and CLI for the Switches.

This approach makes sense because the APs handle the wireless medium (WMM categories), while the switches handle the physical hardware queues (QoS Profiles).

Here is the E2E deployment plan for QoS and Guest Isolation based on the logs and manuals:

1. QoS: Creating "Priority Lanes"

We want to move Corporate traffic into the "Express Lane" (QP6) while Guest traffic stays in the "Best Effort" lane (QP1).

- Switch 1 CLI (The Primary Router): You must map the VLANs to hardware queues. On the 5320, we have 8 queues, with QP8 being the highest.

- XIQ Configuration (For APs): The AP needs to mark the wireless traffic before it hits the switch port.

  - Go to Wireless Networks and edit the Horizon-Corp SSID.

  - Under QoS Settings, set the marking to CoS 5 or 6 (this maps the wireless WMM "Voice" or "Video" categories to the switch's QP6).

  - Set the Horizon-Guest SSID to a lower marking like CoS 0.

--------------------------------------------------------------------------------

2. Guest Isolation: The Two-Pronged Strategy

Guest isolation is tricky because traffic flows in two different directions: sideways (Guest to Guest) and upward (Guest to Corporate).

A. North-South Isolation (Guest → Corporate)

This is a routing decision that happens on Switch 1. We’ll use the edit policy method, which is the "Gold Standard" for EXOS to avoid the attribute errors we saw earlier.

- Switch 1 CLI:

  - Run edit policy GUEST_ISO.

  - In the text editor, enter the following rules:

  - Save and exit (Esc + :wq).

  - Apply it to the Guest VLAN: configure vlan VLAN_30_Guest_Wireless ipaccess-list GUEST_ISO ingress.

B. East-West Isolation (Guest → Guest)

An ACL cannot block this because the traffic never leaves the AP's radio to hit the switch's routing engine. This must be done in the cloud.

- XIQ Configuration:

  - Go to Configure → Network Policies → [Your Policy].

  - Click the SSIDs tab and select your Horizon-Guest SSID.

  - Navigate to Advanced Settings (or Optional Settings).

  - Find the toggle for Client Isolation and set it to Enabled.

  - Save and Push a "Complete Configuration Update" to both APs.

Alex's Summary Table

| Feature | Point of Enforcement | Why? |
| --- | --- | --- |
| Corp Priority | SW1 (QoS Profiles) | Queues traffic during trunk congestion. |
| Guest → Corp | SW1 (ACL) | Blocks traffic at the L3 Gateway. |
| Guest → Guest | APs (Client Isolation) | Prevents "sideways" radio traffic. |

Next Step for our build: Now that we have the isolation strategy mapped, would you like me to write out the final Verification Checklist (specific show commands) to prove a Guest iPhone can ping 8.8.8.8 but is "invisible" to a Corporate laptop?

I need to first deploy the solutions.

Hey! As your fellow student Alex, I've been looking through our lab logs and the April 7 Syntax Lessons, and the short answer is: Yes, absolutely—most of "THIS" could have been done entirely within ExtremeCloud IQ (XIQ).

In fact, the sources explicitly state that for a managed deployment like ours, using XIQ is the preferred enterprise approach because it handles the "intent" and generates the complex syntax for you.

Here is the breakdown of how XIQ could have handled the specific hurdles you just faced in the CLI:

### 1. Guest Isolation (The "Wall")

Instead of manually writing policy files and fighting with the entry keyword and semicolons, you could have configured this in the XIQ GUI:

- North-South (Guest → Corporate): You can set Layer 3 Firewall Rules directly on the Horizon-Guest SSIDpolicy. You would simply add a "Deny" rule for destination 10.10.0.0/24 and 10.20.0.0/24. XIQ then writes and pushes the correct ACL to the APs and switches automatically.

- East-West (Guest → Guest): This is a simple toggle switch in the SSID's "Advanced Settings" or "Optional Settings" called Client Isolation. Turning this on prevents guests from seeing each other without you ever touching a CLI.

### 2. QoS Priority (The "Express Lane")

While we were mapping QoS Profiles (QP1/QP6) in the CLI, XIQ can handle the "Wireless-to-Wired" priority mapping:

- SSID Markings: Inside the Network Policy → QoS Settings, you can assign the Horizon-Corp SSID a higher priority (like CoS 5 or 6) and the Guest SSID a lower priority.

- The Result: When the AP marks that traffic, a switch configured for dot1p examination (which XIQ can also enable via port templates) will automatically drop those frames into the correct hardware queues.

### Why did we use the CLI at all?

Based on our April 7 lessons, we found that while XIQ is great for L2 (VLANs) and AP policies, it sometimes has "black holes" or bugs when dealing with Switch Engine (EXOS) specifically:

- Granular DHCP: XIQ sometimes struggles to push the specific enable dhcp ports... commands required for EXOS.

- L3 Forwarding: Crucial commands like enable ipforwarding vlan Default (The "Karl Rule") are not currently set by XIQ and must be done via CLI after a factory reset.

- Static Routes: There is a known limitation where XIQ cannot push L3 static routes via an L2 interface, which is why we had to manually tell the switch how to find the modem.

Alex's Summary: You were using the CLI for the "Wired Side" (VLAN 10) because that's where the granular control is needed, while XIQ is best for the "Wireless Side" (SSIDs). If you had done the Guest Isolation in XIQ, you wouldn't have hit that "Missing keyword entry" error because the cloud would have written the code for you!

Would you like me to find the specific menu path in XIQ for the Layer 3 Firewall rules so you can try moving that part of the "intent" to the cloud?

Top of Form

33 sources

Bottom of Form